Nanochemistry
Did you know?
How STM pictures are made


The computer connected to the STM makes a grid of the surface of the substance. The tip of the STM scans the surface at points on the grid. The tip records a tiny electrical current, called the 'tunnelling current', at each point. The current changes as the tip moves up and down depending on the atoms present on the surface. A system inside the STM adjusts the tip movement, keeping it at a constant height. The adjustments are recorded and processed by the computer into an image, showing the changes in the surface of the substance.

The pictures show the process: (1) scanning; (2) the original image showing light areas as small adjustments; (3) the processed image changing light areas into heights; (4) the processed image coloured in shades of grey.

| (1) | (2) | (3) | (4) |
| --- | --- | --- | --- |

Producing an STM picture
Reproduced with kind permission from Mike Ross, IBM Almaden Research Center, California USA.

Then the image is coloured to show different features of the atoms.