### Preparation of an organic liquid

### Practice a wide range of experimental techniques in this experiment to synthesise an organic liquid. Watch a video of the whole procedure, plus find technician notes and more resources, at rsc.li/38LiKx6

## Equipment (per group, for five different concentrations of H2O2)

- 1 x beaker 100 cm3 (to dispense the concentrated acid)

- 1 x measuring cylinder, 50 cm3

- 2 x conical flask, 250 cm3

- Mineral wool for plug (or use cotton wool)

- 1 x glass pipette

- 1 x separating funnel and stopper

- 3 x retort stands, bosses and clamps

- 1 x beaker, 250 cm3 (for collection of waste aqueous layer)

- 2 x filter funnel

- 1 x measuring cylinder, 10 cm3

- 2 x spatula

- 1 x filter paper

- 1 x pear shaped flask, 50 cm3 (or round bottomed flask)

- 1 x deionised water in wash bottle

- 1 x full apparatus for distillation which includes:

  - 1 x still head

  - 1 x thermometer

  - 1 x Liebig condenser

  - 1 x receiver

  - 1 x sample tube

  - 1 x beaker to use as water bath (large enough to contain the pear-shaped flask and water)

- 1 x kettle

- 1 x timer or digital stopwatch

- 2-methylpropan-2-ol (DANGER: flammable, corrosive, irritant, may cause drowsiness and dizziness)

- Concentrated hydrochloric acid (needs to be cold: refrigerate and supply just before use, or provide bottles kept in an ice bath) (DANGER: corrosive, irritant)

- 5% sodium hydrogen carbonate solution

- Anhydrous sodium sulfate, solid

- Anti-bumping granules, a quarter of a spatula

- Safety goggles (for first part of the experiment)

- Safety glasses

- Access to a fume cupboard

- Gloves (to handle the mineral wool only)

## Safety

Wear eye protection throughout and gloves when handling the mineral wool. Wear gloves when dispensing concentrated hydrochloric acid if you have an open wounds or skin condition.

Work in a fume cupboard when using concentrated hydrochloric acid.

Keep chemicals away from fire.

## Method

### Part 1: preparation

### Work in a fume cupboard.

- Measure 20 cm3 of cold concentrated hydrochloric acid and pour it in a conical flask.

- Gradually add the 2-methylpropan-2-ol, 1 cm3 at a time. Swirl the conical flask and place a mineral wool plug in the neck of the flask after each addition.

- Leave the flask for 20 minutes to allow the reaction to take place.

### Part 2: separation

Separate the two layers in the conical flask.

- Pour the mixture into a separating funnel using a filter funnel.

- Open the tap to allow the aqueous layer to drain into a beaker. (Watch the video for tips on how to use the separating funnel.)

### Part 3: purification I

- To remove any unreacted hydrochloric acid, add 10 cm3 of sodium hydrogen carbonate solution to the separating funnel.

- Put the stopper back onto the separating funnel, keep your finger on the stopper and invert it.

- Open the tap and swirl the solution (do not shake and make sure the tip is not pointing at anyone).

- Return the separating funnel to the stand and remove the aqueous layer at the bottom of the separating funnel into the waste beaker.

- Repeat steps 6–9, then collect the organic layer in a conical flask.

Purification II

- To remove any trace of water left in the sample, add half a spatula of solid anhydrous sodium sulfate and swirl. Repeat using half a spatula at a time until the sodium sulfate no longer clumps together.

- Filter the mixture into a pear-shaped flask.

Purification III

- Add anti bumping granules to the flask.

- Set up the distillation equipment as shown in the diagram below. Turn on the cooling water (you only need a slow flow through the equipment).

- Add boiled water from a kettle to heat the pear-shaped flask in a water bath to the desired temperature. Collect your sample between 49–52°C into a clean dry test tube.

## Disposal

- Always dispose of chemicals carefully following your teacher’s instructions.

- Any unreacted concentrated hydrochloric acid would have been neutralised by the sodium hydrogen carbonate solution but make sure equipment used to dispense it is rinsed in the fume cupboard.

- Flush the aqueous waste down a foul-water drain with plenty of water.